# Safety Cage SPECIFICATION

# Biesheuvel Autosport is a certified company for installations off roll-bars
This roll bar is built in under Annexe J artikel 253 self construction

| Safety Cage Manufacturer | Biesheuvel Autosport |
| --- | --- |
| Address | Rijksweg 37 |
|  | 4255 GE Nieuwendijk |
|  | Netherlands |
| Phone n° + Email | 0031 183 403400 info@biesheuvel.nl |

The structure shown on this form is manufactured for the following vehicle:

| Make | NISSAN | Model | PATHFINDER | Year/Model Identifier | 2008 |
| --- | --- | --- | --- | --- | --- |
| Vehicle Homologation Number(s)* *if applicable |  |  | N/A |  |  |

Drawing of front ¾ view of the Complete Structure showing the location of the identification plate

6090

DETAILS OF STRUCTURE

|  | A = | 380 | mm | B= |  | mm |
| --- | --- | --- | --- | --- | --- | --- |
|  | C = |  | mm | E = | 330 | mm |
|  | H = | 1140 | mm | (E is highest figure only) |  |  |
| All Dimensions 5mm |  |  |  |  |  |  |

| Method of attachment to Body Shell | Welded |  |
| --- | --- | --- |
| Weight of Safety Cage, inc. fastenings | 25 | kg |

LABELLED DRAWING OF THE SAFETY CAGE STRUCTURE SHOWING ALL TUBING DIMENSIONS

O D S O1 D1 C C1 L/F M E B

| All Dimensions + 0.1mm |  | Show location of each type on drawing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tube / Member | Label | Grade of Steel | External Diameter (OD) |  | Wall Thickness |  | Min. Tensile Strength |  |
| Main Rollbar | M | E235EN/10305-1 | 45.0 | mm | 2.5 | Mm | 480 | N/mm2 |
| Front/Lateral Rollbar | L/F | E235EN/10305-1 | 40.0 | mm | 2.5 | Mm | 480 | N/mm2 |
| Backstays | B | E235EN/10305-1 | 40,0 | mm | 2.5 | Mm | 480 | N/mm2 |
| Main rollbar Diagonal members | D | E235EN/10305-1 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| Backstay Diagonal members | D1 | E235EN/10305-1 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| Roof Reinforcements | C | E235EN/10305 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| Doorbars | E | E235EN/10305-1 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| Windscreen Pillar Reinforcements | C1 | E235EN/10305-1 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| Harness Bar | S | E235EN/10305-1 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| OPTINAL TUBES | O | E235EN/10305-1 | 40.0 | mm | 2.0 | Mm | 480 | N/mm2 |
| OPTINAL TUBES | O1 | DIN2391 25CRM04 | 30.0 | mm | 2.5 | Mm | 700 | N/mm2 |
| OPTINALTUBES |  |  |  | mm |  | Mm |  | N/mm2 |
|  |  |  |  | mm |  | Mm |  | N/mm2 |
|  |  |  |  | mm |  | mm |  | N/mm2 |
|  |  |  |  | mm |  | mm |  | N/mm2 |

| Total length of all tubes < 40 mm OD |  | mm | 15 m maximum |
| --- | --- | --- | --- |

| Photo 1 FRONT ¾ VIEW OF CAGE |  | Photo 2 REAR ¾ VIEW OF CAGE |
| --- | --- | --- |

| Photo 3 VIEW FROM SIDE OF CAR |  | Photo 4 MAIN ROLLBAR TO FLOOR ATTACHMENT |
| --- | --- | --- |

| Photo 5 REAR BACKSTAY TO BODYSHELL ATTACHMENT |  | Photo 6 FRONT ROLLBAR TO FLOOR ATTACHMENT |
| --- | --- | --- |

| Photo 7 FRONT ROLLBAR TO “a” PILLAR ATTACHMENT |  | Photo 8 MAIN ROLLBAR TO “B” PILLAR ATTACHMENT |
| --- | --- | --- |

| Photo 9 ATTACHMENT TO FRONT SUSPENSION |  | PHOTO Chassis Identification Number |
| --- | --- | --- |

| Photo 10 Anti-Intrusion DOORBar - DRIVER |  | Photo 11 Anti-Intrusion DOORBar – CO-DRIVER |
| --- | --- | --- |

| Photo 12 SHOULDER HARNESS ATTACHMENT - DRIVER |  | Photo 13 SHOULDER HARNESS ATTACHMENT – CO DRIVER |
| --- | --- | --- |

| Photo 14 LOCATION OF INDENTIFICATION PLATE |  | PHOTO 16 DETAILS OF INDENTIFICATION PLATE |
| --- | --- | --- |

| Photo 17 OTHER DETAILS OF CAGE |  | Photo 18 OTHER DETIALS OF CAGE |
| --- | --- | --- |

| V.I.N. or Chassis Identification Number | 5N1BR18BX8C610441 |
| --- | --- |
| BABM51040JH28892 | 6090 |

MANUFACTURER'S DECLARATION AND DETAILS:

I declare that the Safety Cage Structure described on this form and fitted to the vehicle described above:

•	was constructed and installed in accordance with the specifications and design shown herein.

•	has been manufactured so that all aspects of the Safety Cage design, including configuration and strength requirements and welded and demountable joints and mountings, will be in conformity with the requirements of the FIA Homologation Regulations for Safety Cages.

•	has been identified by an engraved plate bearing the name or logo of the manufacturer, ( Biesheuvel Autosport)

| Manufacturer’s Name | Biesheuvel Autosport | Insert Manufacturer Stamp here |
| --- | --- | --- |
| Address | Rijksweg 37 |  |
|  | 4255 GE Nieuwendijk |  |
|  | Netherlands |  |
| Phone n° + Email | 0031 183 403400 info@biesheuvel.nl |  |
| Signature: | Insert signature here |  |
| Name of Signatory#: | N Biesheuvel |  |

MANUFACTURER’S AGENT: (For Safety Cages installed by an Agent)

The following Agent is authorised to act on the manufacturer’s behalf to install this Safety Cage design.

| Agent’s Name | GB Metaal Techniek |
| --- | --- |
| Address | Bredesteeg 2 |
|  | 4255 GH Nieuwendijk |
|  | Netherlands |
| Phone n° + Email | 0031 622607952 info@rolbeugels.nl |

Instructions to the installer of the Safety Cage